Name: ®@π@ $@@β

VU da PAWA iko ee RANA

Questions No 01	 							                 20 marks

Write the equivalent assembly language instructions for the given machine language instructions. Write all the conversion steps for all instructions sequence wise.

| Sr # | Machine Language Instruction | Equivalent Assmebly Language Instruction |
| --- | --- | --- |
| 1 | E1C60004 h |  |
| 2 | 61885000 h |  |
| 3 | 724E8000 h |  |
| 4 | 1A4000D4 h |  |
| 5 | 084000D0 h |  |

Step: 1

| Sr # | Given instruction in hexadecimal | Equivalent instruction in binary |
| --- | --- | --- |
| 1 | E1C60004 h | 1110 0001 1100 0110 0000 0000 0000 0100 |
| 2 | 61885000 h | 0110 0001 1000 1000 0101 0000 0000 0000 |
| 3 | 724E8000 h | 0111 0010 0100 1110 1000 0000 0000 0000 |
| 4 | 1A4000D4 h | 0001 1010 0100 0000 0000 0000 1101 0100 |
| 5 | 084000D0 h | 0000 1000 0100 0000 0000 0000 1101 0000 |

Step: 2

| Sr # | Given instruction in hexadecimal | Op-code field | mnemonic |
| --- | --- | --- | --- |
| 1 | E1C60004 h | 1110 0 b | shl |
| 2 | 61885000 h | 0110 0 b | add |
| 3 | 724E8000 h | 0111 0 b | sub |
| 4 | 1A4000D4 h | 0001 1 b | st |
| 5 | 084000D0 h | 0000 1 b | ld |

Step: 3

| Sr # | Given instruction in hexadecimal | mnemonic | Instruction type |
| --- | --- | --- | --- |
| 1 | E1C60004 h | shl | C |
| 2 | 61885000 h | add | D |
| 3 | 724E8000 h | sub | D |
| 4 | 1A4000D4 h | st | C |
| 5 | 084000D0 h | ld | C |

Step: 4

| Sr # | Machine Language Instruction | Equivalent Assmebly Language Instruction |
| --- | --- | --- |
| 1 | E1C60004 h | Shl R7, R3, 4 |
| 2 | 61885000 h | Add R6, R4, R5 |
| 3 | 724E8000 h | sub R9, R7, R8 |
| 4 | 1A4000D4 h | st R9, z |
| 5 | 084000D0 h | ld R1, c |