Cs601

Lab lacture 101

Transmission Media:
It is the physical path used to transfer data signals between devices in a network.

Explanation:

- It carries data from one device to another.

- It operates below the physical layer of the OSI model (Layer 0).

- Types: Metallic (e.g., twisted pair, coaxial cable), Optical (e.g., fiber optics), and Free Space (e.g., wireless signals through air or vacuum).

Transmission Media

- Positioned below the Physical Layer (Layer 0).

- Directly managed by the Physical Layer.

- Types include:

  - Metallic Media:

    - Twisted Pair Cable.

    - Coaxial Cable.

  - Optical Media:

    - Optical Fiber Cable.

  - Free Space:

    - Air or Vacuum (used in wireless communication).

These media are responsible for carrying data signals.

Guided Media:
Guided media are physical transmission paths where signals are confined to a specific medium.

Examples:

- Twisted Pair Cable: Two insulated wires twisted together.

- Coaxial Cable: A single conductor surrounded by insulation and shielding.

- Fiber Optic Cable: Uses light to transmit data through thin glass or plastic fibers.

Explanation:

- Signals travel through a guided path.

- Used for wired communication like LANs and internet cables.

Twisted-pair cable consists of two copper wires, each with its own plastic insulation, twisted together.

- Signal wire: Carries the data signals.

- Ground reference wire: Acts as a reference for the signal.

Twisted-Pair Cable comes in two types: UTP (Unshielded Twisted Pair) and STP (Shielded Twisted Pair).

UTP (Unshielded Twisted Pair):

  - No additional shielding around the wires.

  - More affordable and commonly used for networking (e.g., Ethernet).

  - Susceptible to interference and crosstalk.

STP (Shielded Twisted Pair):

  - Has a shield (metallic layer) around the twisted wires to protect from external interference.

  - Provides better protection against noise and crosstalk.

  - More expensive and used in environments with high electromagnetic interference (EMI).

Coaxial cable carries higher frequency signals than twisted-pair cables. It has a central copper conductor, insulation, a shield to block interference, and an outer layer. This structure helps reduce noise and allows for better signal quality over longer distances, making it good for TV, internet, and data transmission.

BNC (Bayonet Neill-Concelman) connectors are used to connect coaxial cables. They are commonly used in video and data applications. BNC connectors have a simple twist-lock mechanism that ensures a secure connection. They are often used in applications like CCTV, radio frequency (RF) systems, and networking.

Coaxial cable performance depends on its ability to carry signals without significant loss or interference. Key factors include:

- Bandwidth: Coaxial cables can support higher frequencies than twisted pair cables, allowing faster data transmission.

- Signal Integrity: The shield in coaxial cables protects the signal from external interference (noise), improving the quality of data transmission.

- Attenuation: Signal loss over distance is called attenuation. Higher-quality coaxial cables have lower attenuation, allowing signals to travel longer distances without degradation.

- Impedance Matching: Proper impedance matching (usually 75 ohms) ensures minimal signal reflection, maintaining transmission quality.

Coaxial cables are ideal for high-frequency applications, such as cable TV, internet connections, and some networking setups.

Fiber-optic cables are made of glass or plastic fibers and carry signals as light pulses. The core of the fiber carries the light signals, while the cladding (outer layer) reflects the light back into the core, preventing signal loss.

Fiber-optic cables are made of glass or plastic fibers that carry data as light. The core carries the light signals, and the outer layer (cladding) keeps the light inside by bouncing it back.

Key points:

- Total Internal Reflection: The light bounces inside the fiber, which helps it travel long distances without losing signal.

- Fast Speed: Light moves quickly, so fiber-optic cables are great for fast internet and long-distance communication.

- No Interference: These cables don't get disturbed by electrical signals, making them more reliable than metal cables.

Fiber-optic cables are great for fast, clear, and long-distance data transfer.

Fiber-optic cables are made of glass or plastic and carry data in the form of light signals. They allow for fast, long-distance communication with less signal loss and no interference from electrical signals. These cables are used in high-speed internet, phone lines, and other data transfer applications.

The diagram you shared shows the classification of modes in fiber optic communication:

Single Mode:

  - Allows only one light path.

  - Used for long distances and high-speed communication.

Multimode:

  - Allows multiple light paths.

  - Subtypes:

    - Step Index: Light reflects sharply in the core.

    - Graded Index: Light bends gradually, reducing distortion.

These Advantages of Fiber Optics:

- Higher Bandwidth: Supports more data transmission.

- Less Attenuation: Signals travel farther without losing strength.

- Less EM Interference: Immune to electromagnetic interference.

- Light Weight: Easier to handle and install.

- Less Corrosive: More durable than copper cables.

Disadvantages of Fiber Optics:

- Installation/Maintenance: Requires skilled labor and tools.

- Unidirectional: Needs separate fibers for sending and receiving.

- Cost: Higher initial cost compared to copper cables.

affect how data signals propagate through optical fibers.

Unguided Media refers to communication where signals are transmitted through the air without a physical medium. It is commonly used for wireless communication.

Unguided Media refers to communication where signals are transmitted through the air without a physical medium. It is commonly used for wireless communication.

Types:

Radio Waves:

  - Used for TV, AM/FM radio, and mobile communication.

  - Covers long distances and penetrates buildings.

Microwaves:

  - Used for satellite and point-to-point communication.

  - Requires a clear line of sight.

Infrared (IR):

  - Used for short-range communication (e.g., remote controls).

  - Cannot pass through walls.

Advantages:

- No cables needed.

- Suitable for large areas.

Disadvantages:

- Prone to interference.

- Limited security.

- Radio Waves: Electromagnetic waves with frequencies between 3 kHz and 1 GHz. Used for radio, TV, and other wireless communication.

- Microwaves: Electromagnetic waves with frequencies between 1 GHz and 300 GHz. Used for satellite communication, radar, and microwave ovens.

The Electromagnetic Spectrum is a range of waves, from low to high frequencies:

- Radio Waves: Used for radio and TV.

- Microwaves: Used for satellites and cooking.

- Infrared: Heat and remote controls.

- Visible Light: Light we can see.

- Ultraviolet: Sunlight and sterilization.

- X-rays: Used in medical imaging.

- Gamma Rays: Used in cancer treatment.

An Omnidirectional Antenna is an antenna that radiates signals in all directions evenly, forming a 360-degree pattern. It is commonly used in devices like cell phones and Wi-Fi routers to provide coverage in every direction.

Microwaves are electromagnetic waves with frequencies between 1 and 300 GHz.

- They are unidirectional, meaning they travel in one direction.

- When transmitted by an antenna, they can be narrowly focused for precise communication.

The Electromagnetic Spectrum is the range of all electromagnetic waves, ordered by frequency and wavelength:

- Radio Waves: Low frequency, used for radio, TV, and communication.

- Microwaves: Higher frequency, used for satellites, radar, and cooking.

- Infrared: Heat radiation, used in remote controls and night vision.

- Visible Light: Light we can see.

- Ultraviolet: Beyond visible light, used in sterilization.

- X-rays: Used for medical imaging.

- Gamma Rays: Highest frequency, used in cancer treatment.

Each type has a different wavelength and frequency.

Unidirectional Antennas focus signals in one direction, providing stronger transmission and reception in that specific direction. They are commonly used for long-distance communication, like satellite or point-to-point links.

Infrared Waves:

- Frequencies: 300 GHz to 400 THz, with wavelengths from 1 mm to 770 nm.

- Short-range Communication: Used for devices like remote controls and short-distance data transfer.

- Cannot Penetrate Walls: Limited to line-of-sight communication.

- Prevents Interference: Helps avoid interference between different systems in close proximity.

Switching is a method used to connect devices in a network for one-to-one communication.

- A network is a group of connected devices.

- Switching helps connect multiple devices so they can communicate with each other.

- A switched network uses a series of switches to manage the communication paths between devices.

A Switched Network is a type of network where devices communicate through a series of switches.

- Switches manage data traffic by directing signals to the correct destination.

- It allows on-demand connections between devices for one-to-one communication.

- The network can be dynamic, creating and breaking connections as needed.

Common examples include telephone networks and the internet.

The three methods of switching are:

- Circuit Switching: Establishes a dedicated communication path between devices for the duration of the call (used in traditional phone systems).

- Packet Switching: Breaks data into packets and sends them independently through the network (used in the internet and modern communication systems).

- Message Switching: Entire messages are stored and forwarded from one device to another. It's rarely used today.

Switched Networks Taxonomy:

- Circuit Switching: Fixed path for communication (e.g., phone calls).

- Packet Switching: Data sent in packets via different routes (e.g., internet).

- Message Switching: Entire message stored and forwarded (e.g., old telegrams).

Used to manage data flow in networks.

Circuit-Switched Networks:

- A network of switches connected by physical links.

- A dedicated path is established between two stations.

- Each connection uses one dedicated channel on each link.

- Links are divided into channels using FDM (Frequency Division Multiplexing) or TDM (Time Division Multiplexing).

It ensures continuous and reliable communication during the connection.

Circuit-Switched Networks:

- A system where switches are connected by physical links.

- Establishes a dedicated path for communication between two stations.

- Ensures reliable and continuous data flow.

In a circuit-switched network, there are three phases:

- Connection Setup: The network creates a path for communication.

- Data Transfer: Data is sent through the established path.

- Connection Teardown: The connection is closed, and resources are freed.

Circuit-switched networks are less efficient than packet-switched networks because:

- Dedicated resources are reserved for the entire duration of the connection, even if no data is being transferred.

- These resources cannot be used by other connections during this time, leading to wasted capacity.

In a telephone network, resources are tied up for the whole call, even if the person isn't speaking, leading to wasted capacity.

In a data network, this becomes a bigger problem because:

- The connection might stay open even when no data is being sent, wasting resources.

- This makes the network less efficient when there’s no active communication.

Circuit-switched networks have minimal delay because data flows without stopping once the connection is set up. However, they have low efficiency since resources are reserved for the entire connection, even when not in use.

In a circuit-switched network, delay happens only during the setup phase, when the dedicated path is being established between the sender and receiver. Once the connection is set, there is minimal delay during data transfer because the path is fixed and resources are reserved.

In a packet-switched network, the message is broken into packets of fixed or variable size. The network and protocol decide the packet size. These packets travel separately and are reassembled at the destination.

In datagram networks, each packet is treated separately. Even if it’s part of a larger message, the network doesn’t link it with others. Each packet is called a datagram and is sent independently, possibly taking different routes to the destination.

A virtual-circuit network combines features of both circuit-switched and datagram networks. It sets up a temporary path like a circuit-switched network but treats packets like in a datagram network. Packets follow the same route, and the connection is established before data is sent.

A virtual-circuit network is like a mix of circuit-switched and datagram networks. It first sets up a temporary path for the data, like a circuit-switched network, but sends the data in packets like a datagram network. All packets follow the same path during the session.

A circuit switch can use two types of technologies:

Space-Division Switch: This uses physical separate paths for each connection. Each call or data flow gets its own dedicated path.

Time-Division Switch: This shares the same path but divides the time into slots. Each call or data flow gets its turn in the time slots.

A crossbar switch with 3 inputs and 4 outputs connects inputs to outputs in a grid-like pattern. It has 3 input lines and 4 output lines, with a matrix of crosspoints where each input can connect to any output.

- 3 inputs can be connected to any of the 4 outputs, but only one connection can be made at each crosspoint.

- This setup allows flexible routing but requires multiple crosspoints to make all possible connections.

A multistage switch has multiple stages of switches. Data passes through different stages before reaching the output, making it more efficient for handling many connections.

A Time-Division Switch uses TDM (Time-Division Multiplexing) inside the switch to share the same path by dividing time into slots. The most common method is Time-Slot Interchange (TSI), which rearranges time slots for efficient data transfer.

Ppt 9

The Data-Link Layer helps devices (like routers or switches) talk to each other on a network. When a packet moves from one host to another, it passes through these devices, and the Data-Link Layer makes sure the packet is sent correctly between them.

The Internet is made up of many networks connected by devices like routers or switches. When a packet travels from one host to another, it passes through these networks. The Data-Link Layer controls the communication between devices on the same network, ensuring the packet moves correctly from one device to the next.

At the Data-Link Layer, communication happens between devices on the same network. It handles the **fr

In the Data-Link Layer, communication is node-to-node. When data moves from one point on the Internet to another, it passes through many networks (like LANs and WANs). The end hosts and routers are called nodes, and the networks in between are called links.

In networking, nodes are devices like computers, routers, or switches that send or receive data. Links are the connections between these nodes, such as cables or wireless signals. Data travels through these links from one node to another.

The Data-Link Layer is between the Physical Layer and the Network Layer. It provides services to the Network Layer and gets services from the Physical Layer.

Its key services include:

- Framing: It packages data into frames for easy transmission.

- Flow Control: It manages the speed of data transmission to prevent overload.

- Error Control: It detects and corrects errors in data transfer.

- Congestion Control: It helps manage network congestion by controlling data flow.

In communication with three nodes, data can flow between each pair of nodes. For example:

- Node A sends data to Node B.

- Node B sends data to Node C.

- Node A can also send data to Node C through Node B, depending on the network setup.

The nodes are connected via links, and the data may pass through intermediate nodes (like routers or switches) for routing.

The Data-Link Layer helps the Network Layer and uses the Physical Layer. It does the following:

- Framing: It breaks data into smaller pieces called frames.

- Flow Control: It controls how fast data is sent to avoid overloading.

- Error Control: It checks and fixes errors in the data.

- Congestion Control: It helps manage traffic to avoid network jams.

There are two types of links in the Data-Link Layer:

Full Capacity Link: The data-link layer uses the entire link for sending data.

Shared Capacity Link: The data-link layer only uses part of the link because the link is shared with others. It manages how data is sent to avoid problems.

There are two types of links:

Point-to-Point Link: A direct connection between two devices.

Broadcast Link: A shared connection where multiple devices can send and receive data.

The Data-Link Layer has two sublayers:

Data Link Control (DLC): This handles error detection, flow control, and the framing of data.

Media Access Control (MAC): This controls access to the shared communication medium, deciding which device can send data at a given time.

The Data-Link Layer is divided into two sublayers:

Data Link Control (DLC): Manages data transmission, ensuring error detection, framing, and flow control.

Media Access Control (MAC): Controls how devices access and use the shared communication medium, like deciding who can send data and when.

Link-layer addressing is needed because IP addresses only tell us where the source and destination are, but not how the packet travels between them. The Link-layer address (like MAC addresses) helps guide the packet through the network links, so it can reach its destination correctly.

IP addresses are used to find devices across different networks on the network layer.

Link-layer addresses (like MAC addresses) are used to identify devices on the same network or link, helping the packet reach the right device within that network.

There are three types of link-layer addresses:

- Unicast: Sends data from one device to one specific device.

- Multicast: Sends data from one device to a group of devices.

- Broadcast: Sends data from one device to all devices on the network.

An example of a unicast link-layer address in Ethernet is a 48-bit address, written as 12 hexadecimal digits separated by colons. For example:

A3:34:45:11:92:F1

In this address, the second digit (3) is an odd number, which is typical in Ethernet addresses for certain network identification purposes.

A multicast link-layer address in Ethernet is a 48-bit address, written as 12 hexadecimal digits separated by colons. For multicast, the second digit must be an even number.

For example:
A2:34:45:11:92:F1

Here, the second digit (2) is even, which follows the rule for multicast addresses in Ethernet.

A broadcast link-layer address in Ethernet is a 48-bit address where all bits are set to 1. It is used to send data to all devices on the network.

Example: FF:FF:FF:FF:FF:FF

Address Resolution Protocol (ARP) helps a device find the link-layer address (like a MAC address) of another device on the same network. The device already knows the IP address but needs the link-layer address to send data over the network.

We need Address Resolution Protocol (ARP) because knowing only the IP address of a device is not enough to send data over a network. ARP helps find the link-layer address (like the MAC address) of the device, so data can be delivered on the same network.

ARP is used when a device wants to send data to another device using its IP address. But since the IP address can't send data directly over the link, ARP helps find the MAC address (link-layer address) of the next device, so the data can be delivered correctly on the network.

OR

ARP helps a device find the MAC address of another device when it knows only the IP address. This is needed to send data correctly over the network.

ARP is used when a device wants to send data to another device on the same network. The device knows the IP address but needs the MAC address (a unique address for the device) to send the data. ARP helps find the MAC address so the data can be sent correctly.

In our example of the internet:

- A device needs to find the MAC address of the next device (like a router) to send data.

- It uses ARP to ask for the MAC address using the IP address.

- Once it has the MAC address, it sends the data to that device.

- The data keeps moving from device to device until it reaches the destination.

This helps the data travel correctly through the network.

Ppt 10

Types of Errors:

Single-bit Error: One bit in the data changes from 1 to 0 or from 0 to 1.

Interference: Outside signals or noise can mess with the data, causing errors.

These errors happen because something disrupts the data being sent.

A Burst Error happens when 2 or more bits in the data change at once, from 1 to 0 or 0 to 1.

Single-bit error: Only 1 bit in the data changes from 1 to 0 or 0 to 1.

Burst error: 2 or more bits in the data change from 1 to 0 or 0 to 1 at the same time.

Redundancy means adding extra bits to the data. These extra bits help the receiver detect or fix errors in the data.

Detection vs. Correction:

- Error Detection: We only check if there’s an error (yes or no).

- Error Correction: It’s harder because we need to fix the errors.

In detection, a single-bit error and a burst error are both just seen as errors.

**Single-bit error**: Only 1 bit in the data changes (e.g., from 0 to 1 or 1 to 0).

**Burst error**: 2 or more bits in the data change (e.g., from 0 to 1 or 1 to 0).

**Redundancy** is key for detecting and correcting errors. We add extra bits to our data. These extra bits help the receiver find and fix any errors, ensuring the data is accurate and reliable.

Detection vs. Correction:

Detection: Easier; checks if an error occurred (Yes/No). Doesn't count corrupted bits.

Correction: Harder; fixes the errors.

Single-bit error: One bit changes.

Burst error: Multiple bits change.

Detection vs. Correction:

Detection: Checks if an error occurred (Yes/No). Doesn't count corrupted bits.

- Correction: Identifies the exact number and location of corrupted bits to fix them

- Coding: Redundancy is achieved using different coding methods. The sender adds extra bits that relate to the actual data bits. The receiver then checks these relationships to find and fix errors.

- **Coding**: The ratio of extra (redundant) bits to actual data bits and the strength of the coding process are crucial. These factors determine how well errors can be detected and corrected.

Types of Coding Schemes:

Block Coding: Data is divided into fixed-size blocks, and extra bits are added to each block to detect and correct errors.

Convolution Coding: Data is encoded using overlapping segments, creating a continuous stream of coded bits that help detect and correct errors.

These methods ensure data is transmitted accurately and reliably.

Block Coding: We split our message into blocks of 'k' bits, called datawords. We then add 'r' extra bits to each block, making the total length 'n = k + r'. These new 'n-bit' blocks are called codewords.

In block coding:

- We split the message into small parts called datawords, each with k bits.

- We add r extra bits (redundant bits) to each dataword.

- This makes the total length n = k + r.

- These new n-bit parts are called codewords.

This helps in detecting or correcting errors.

In block coding for error detection:

- The receiver knows or can find a list of valid codewords.

- If the original codeword changes to an invalid codeword, the receiver can detect the error.

Block coding:

- Split the message into small parts called datawords with k bits.

- Add r extra bits (redundant bits) to each part.

- Each part becomes n bits long, where n = k + r.

- These n-bit parts are called codewords.

Example:
If k = 2 and n = 3:

- Dataword has 2 bits (like 00, 01, 10, 11).

- Codeword adds 1 extra bit (r = n - k = 1) to make it 3 bits (like 000, 011, etc.).

We use a rule or method to create the codeword from the dataword. This ensures errors can be detected later.

Hamming Distance:

- It counts the number of different bits between two same-size words.

- For two words x and y, it is written as d(x, y).

- The Hamming distance between a sent codeword and a received codeword shows how many bits got changed (corrupted) during transmission.

Example:
Sent: 1101
Received: 1001
Hamming distance = 1 (only 2nd bit is different).

If a code has Hamming distance (dmin) = 4:

- It can detect up to 3 errors.

Concept:
To detect errors, the number of errors (e) must be less than or equal to dmin - 1.
Here, e ≤ 4 - 1 = 3.

So, this code can detect 1, 2, or 3 errors.

Linear Block Codes:

- They are a type of block code.

- In linear block codes, if you take two valid codewords and perform XOR (exclusive OR) on them, the result will be another valid codeword.

Concept:
XOR is a bitwise operation. If two codewords are valid, XOR-ing them keeps the result valid. This property makes error detection and correction easier.

Example of Linear Block Code:

- In this code, XORing any two codewords gives another valid codeword.

- For example, if you XOR the second and third codewords, you get the fourth codeword.

- The number of 1s in each nonzero codeword is 2.

- The minimum Hamming distance (dmin) is 2, meaning the code can detect up to 1 error.

Concept:

- Linear block codes have the property that combining (XOR-ing) any two valid codewords results in another valid codeword.

- The minimum Hamming distance tells us how many errors the code can detect. Here, dmin = 2, so it can detect 1 error.

Simple Parity-Check Code C(5, 4):

- This code has 5 bits in total: 4 data bits and 1 parity bit.

- The parity bit makes sure the total number of 1s in the codeword is even.

Example:
For data 1011, there are 3 ones. To make it even, we add 1 as the parity bit, making the codeword 10111.

Encoder for Simple Parity-Check Code:

- Take 4 data bits (e.g., 1011).

- Count how many 1s there are.

- Add a parity bit to make the total number of 1s even.

  - If the count is odd, add 1.

  - If the count is even, add 0.

- Now, the codeword is 5 bits (e.g., 10111).

Decoder for Simple Parity-Check Code:

- Receive the 5-bit codeword (e.g., 10111).

- Count the number of 1s.

- If the count is odd, an error has occurred.

- If the count is even, no error was found.

Cyclic Codes:

- These are special linear block codes.

- The key feature is that if you rotate (cyclic shift) a codeword, the result is still a valid codeword.

Example:

- If 1011000 is a codeword, then rotating it (shifting left) gives 0110001, which is also a valid codeword.

- This rotation property makes it easier to detect and correct errors.

Cyclic Redundancy Check (CRC):

- CRC is a method used to detect errors in data.

- It's used in networks like LANs and WANs.

How it works:

- The sender adds a CRC code to the data before sending it.

- The receiver checks the data using the same CRC.

- If the check is correct, the data is fine; if not, there’s an error.

A CRC code with C(7, 4) means:

- The code has 7 bits in total: 4 data bits and 3 CRC bits.

- 4 data bits are sent, and the 3 CRC bits help detect errors.

How it works:

- The sender sends 4 data bits and adds 3 CRC bits to make it a total of 7 bits.

- The receiver checks the 7 bits to see if the CRC is correct. If it’s wrong, an error is detected.

Cyclic Redundancy Check (CRC):

- CRC is a method used to check for errors in data.

- It’s part of cyclic codes and helps ensure data is transmitted correctly.

- CRC is widely used in networks like LANs (Local Area Networks) and WANs (Wide Area Networks).

How it works:

- Before sending data, a CRC code is added to it.

- The receiver checks the data using the same CRC. If there’s an error, the receiver can detect it.

CRC Encoder:

- The sender takes the data and adds a special CRC code to it.

- This code is calculated using a formula (called the generator polynomial).

- The sender then sends the data with the CRC code attached.

CRC Decoder:

- The receiver gets the data with the CRC code.

- The receiver uses the same formula to check the data.

- If the result is zero, the data is correct. If not, there was an error during transmission.

In simple terms:

- The encoder adds a check to the data before sending it.

- The decoder checks the data when it arrives to see if there’s an error.

Cyclic Redundancy Check (CRC):

- CRC is used to check for errors in data.

- It's used in networks like LANs and WANs to make sure data is correct.

How it works:

- The sender adds a CRC code to the data.

- The receiver checks the data with the CRC code.

- If the check is correct, the data is fine; if not, there’s an error.

In simple terms:
CRC helps ensure that data sent through a network has no mistakes. If there’s a mistake, it can be found.

In simple terms:

- Cyclic codes use math equations to check if data has errors.

- The formula helps add an error check (CRC). If it doesn't match, an error occurred.

Advantages of Cyclic Codes:

Good at finding errors:

  - Detects if one bit is wrong.

  - Detects if two bits are wrong.

  - Finds errors in an odd number of bits.

  - Catches errors in a group of bits (burst errors).

Easy to use:

  - Simple to set up and work with.

Fast:

  - It checks errors quickly, making it efficient for sending data.

Checksum:

- A checksum is a method to check if data has errors.

- It can be used for messages of any size.

- Checksum is mostly used in the network and transport layers instead of the data-link layer.

How it works:

- The sender adds the data together to create a checksum (a small number).

- The receiver adds the data and checks if it matches the checksum. If they match, the data is correct; if not, there's an error.

Concept Behind Checksum:

- A checksum helps check if the data sent is correct.

- It works by adding up all the data and sending the total (checksum) with it.

- The receiver adds up the data again and checks if the total matches the checksum. If it matches, the data is correct; if not, there’s an error.

Simple Example:

Sender:

  - Data: 5, 10, 3

  - Add them up: 5 + 10 + 3 = 18

  - The sender sends the data (5, 10, 3) and the checksum 18.

Receiver:

  - Data received: 5, 10, 3

  - Add them up: 5 + 10 + 3 = 18

  - The receiver compares the sum to the checksum: 18 matches 18, so the data is correct.

If the numbers didn’t match, an error would have occurred.

Checksum Concept:

Checksum checks if data is received correctly.

How it works:

Sender:

  - Adds all data values.

  - Sends data + the sum (checksum).

Receiver:

  - Adds received data values.

  - Compares with the checksum.

If they match: Data is correct.
If not: There’s an error.

Example: Data = [5, 10, 15]
Checksum = 5+10+15=305 + 10 + 15 = 30

Sent: [5, 10, 15] + 30
Received: [5, 10, 15] + 30
If sums match, no errors!

Here’s how checksum works in your example:

Message Numbers: (7,11,12,0,6)(7, 11, 12, 0, 6)
Step 1: Add the numbers:

7+11+12+0+6=367 + 11 + 12 + 0 + 6 = 36

Step 2: Send the message and checksum:

Message=(7,11,12,0,6)and Checksum = 36Message = (7, 11, 12, 0, 6) \quad \text{and Checksum = 36}

Step 3: Receiver checks by adding:
Receiver adds 7+11+12+0+6=367 + 11 + 12 + 0 + 6 = 36.

Step 4: Compare:
If the calculated sum matches the checksum (36), the message is correct.
If not, there's an error.

Here's how the example works step by step:

Decimal Sum:
The sum of the numbers is 3636. In binary, 3636 is (100100)2(100100)_2.

Split Binary into 4 Bits:
Split (100100)2(100100)_2 into two parts:

  - Leftmost 2 bits: (10)2(10)_2

  - Rightmost 4 bits: (0100)2(0100)_2

Add the Parts Together:
Add (10)2+(0100)2(10)_2 + (0100)_2:

- (10)2+(0100)2=(0110)2(10)_2 + (0100)_2 = (0110)_2

Final 4-bit Checksum:
The checksum becomes (0110)2=6(0110)_2 = 6 in decimal.

This technique ensures the checksum is a 4-bit value.

Forward Error Correction (FEC) - Simple Concept:

What is FEC?
A method to detect and correct errors in data during transmission without retransmission.

Why Use FEC?

  - Useful for real-time applications like video calls and streaming.

  - Saves time by correcting errors instantly.

How Does FEC Work?

  - Adds extra data (redundancy) to the message.

  - Receiver uses this extra data to fix errors directly.

Examples of FEC Techniques:

  - Hamming Code: Fixes single-bit errors.

  - Reed-Solomon Code: Used in CDs, DVDs.

  - Convolutional Codes: Used in mobile networks.

Example:

If "1011" is sent with redundancy but received as "1111," FEC corrects it back to "1011."

Hamming Distance in Error Correction:

Hamming Distance:

  - Measures the number of bit differences between two binary codes.

  - Example: Distance between 1011 and 1001 is 1.

For Error Correction:

  - To fix t errors, we need dmin = 2t + 1 (minimum Hamming distance).

  - Example: To fix 10 errors, dmin = 21.

Why Extra Bits?

  - Extra (redundant) bits are added to detect and fix errors.

  - More errors to fix = More bits needed.

To correct 10 errors in a packet using Hamming distance, we need a minimum Hamming distance of 21.

This means there must be at least 21 different bit positions between valid codewords, ensuring the receiver can detect and correct up to 10 errors. The more errors you want to correct, the larger the Hamming distance needs to be, but this also increases the number of redundant bits sent with the data.

XOR (Exclusive OR) is a simple operation used to detect and correct errors in data communication. It compares two bits:

- If they are the same, the result is 0.

- If they are different, the result is 1.

It helps to check if data has changed during transmission. If the result doesn't match what’s expected, it means an error occurred. XOR is used in error detection methods like parity checks and Hamming codes.

Using XOR, you can detect errors by comparing the bits.

- XOR gives 0 for matching bits and 1 for different bits.

- If you XOR all data bits (P1, P2, ... PN) and get a result (R), you can check if there's an error.

- If the result doesn't match what’s expected, there's an error in the data.

This method helps detect errors during data transmission.

Chunk interleaving is a technique used in Forward Error Correction (FEC) for multimedia data. Here's how it works in simple terms:

- Instead of sending a whole packet at once, the data is divided into smaller chunks.

- These chunks are interleaved, meaning they are spread out across multiple packets.

- If some chunks are missing during transmission, only a small part of the data is lost, rather than the entire packet.

- This method helps ensure that even if a chunk is lost, the receiver can still recover the data by using the remaining chunks from other packets.

It balances error correction with efficient data transmission.

Combining Hamming Distance & Interleaving:

- Hamming Distance helps detect and fix errors by ensuring enough difference between data.

- Interleaving mixes data into rows, sending bits column by column to spread errors.

- Together, they help fix many errors, especially burst errors, by correcting m × t errors (m = rows, t = errors per packet).

In short: Hamming corrects errors, and interleaving helps handle burst errors.

Compounding High & Low Resolution Packets:

- We create duplicate low-resolution versions of high-resolution packets.

- The low-resolution packets are sent along with the high-resolution packets.

- This method helps to maintain some quality even if high-resolution packets are lost.

For example: Out of five high-res packets, create four low-res duplicates and send them together.

This way, if high-res packets are lost, low-res ones can still provide a basic level of information.

Compounding High-and-Low Resolution Packets:

- High-resolution packets are sent alongside low-resolution versions.

- If a high-res packet is lost, the low-res version helps provide some information.

- This method improves reliability by balancing between quality and redundancy.

- Ppt11

Data Link Control (DLC):

- Manages communication between two directly connected devices.

- It includes:

  - Framing: Organizing data into frames.

  - Flow Control: Prevents overload by managing data speed.

  - Error Control: Detects and fixes errors in data transmission.

DLC (Data Link Control) Services:

Framing: Organizes data into manageable units (frames) for transmission.

Flow Control: Ensures data is sent at a rate the receiver can handle, preventing loss or overload.

Error Control: Detects and corrects errors in the transmitted data to maintain data integrity.

These services are essential for reliable data transfer between devices.

Framing:

Framing is how data is packaged into frames for transmission. It adds information like sender and receiver addresses, helping the receiver know where a message starts and ends. This makes data easier to send and receive correctly.

Framing:

Framing adds a destination address (where the packet goes) and a sender address (for acknowledgment) to organize data for transmission.

Frame Size:

Fixed Size: The frame always has the same size, making it easy to tell where one ends and the next begins.

Variable Size: The frame size can change. To know where a frame starts and ends, we need special markers or signals.

Variable Framing Techniques:

Byte-Counting: The number of bytes in a frame is counted. The receiver knows where the frame ends by this count.

Flag-Byte Framing: Special flag bytes mark the start and end of the frame. The receiver looks for these flags to know where the frame begins and ends.

Escape Sequence: If a flag appears inside the data, it's replaced with an escape sequence to avoid confusion.

These techniques help in identifying frames when their size isn't fixed.

In a Character-Oriented Protocol, a frame consists of characters (or bytes) that are used to represent data.

Structure of the frame:

- Start Flag: A special character that marks the beginning of the frame.

- Data: The actual message or information being sent.

- End Flag: A special character that marks the end of the frame.

- Control Characters: Optional, used for flow control, error checking, or other management tasks.

The frame uses these characters to separate the data and indicate where it starts and ends. The receiver looks for these flags to correctly interpret the frame.

In Character-Oriented Framing, data is sent as 8-bit characters (1 byte). Each frame has:

- Start Flag: Marks the beginning.

- Data: 8-bit characters.

- End Flag: Marks the end.

It helps organize data into manageable chunks.

Byte Stuffing is used to prevent confusion when data includes characters that match the flag.

- If a data byte is the same as the flag, an escape byte is added before it.

- The receiver removes the escape byte to correctly interpret the data.

This ensures flags aren't mistaken for data.

Byte Stuffing Issues:

- When the data contains a byte that looks like a flag, we use an escape character before it to prevent confusion.

- Unicode (16/32 bit) vs 8-bit characters: Unicode requires more care because it uses more than 8 bits.

- The escape character is predefined to signal that the next byte is data, not a flag.

This ensures the data is accurately interpreted without misreading flags as part of the data.

Byte Stuffing Issues:

- When the data contains a byte that looks like a flag, we use an escape character before it to prevent confusion.

- Unicode (16/32 bit) vs 8-bit characters: Unicode requires more care because it uses more than 8 bits.

- The escape character is predefined to signal that the next byte is data, not a flag.

This ensures the data is accurately interpreted without misreading flags as part of the data.

Byte Stuffing and Unstuffing:

- Byte Stuffing: If data has a flag (like 0x7E), we add an escape byte (0x7D) before it.

- Byte Unstuffing: The receiver removes the escape byte to get the original data.

This prevents confusion between data and flags.

Bit-Oriented Framing:

- The data part of the frame is just bits.

- To separate frames, a special 8-bit pattern (01111110) is used as the start and end delimiter.

- Just like connection-oriented framing, we face the problem of the delimiter appearing in the data.

Bit-Oriented Framing:

- Data is in bits.

- A special pattern (01111110) marks the start and end of a frame.

- We avoid confusion if the pattern appears in data.

Frame in a Bit-Oriented Protocol:

- The frame starts and ends with a special 8-bit pattern (01111110).

- The data part is made up of bits.

- To avoid confusion, if the pattern (01111110) appears in data, bits are added to ensure it doesn't look like a frame delimiter.

Bit Stuffing:

- Add a 0 after five consecutive 1s in data to avoid confusion with the frame boundary.

Bit Unstuffing:

- Remove the added 0 after five consecutive 1s when receiving the data.

Flow Control:

- Manages the speed of data transmission between sender and receiver to prevent data loss.

- Ensures the receiver can handle the incoming data rate.

Error Control:

- Detects and corrects errors during data transmission.

- Methods include checksums, parity, and retransmissions to ensure data integrity.

Flow Control:

- Ensures data is produced and consumed at a balanced rate.

- If data is sent faster than the receiver can process, frames are discarded.

- Buffers are used to temporarily hold data at both the sending and receiving ends to prevent overflow and loss.

Flow Control:

- It makes sure data is sent at a pace the receiver can handle.

- Buffers store data temporarily to avoid loss when the receiver is slow.

Flow Control:

- Manages the speed of data transfer.

- Prevents data loss by making sure the receiver can handle the incoming data.

- Uses buffers to store data temporarily.

- Or

Flow Control at the Data Link Layer:

- It ensures that data frames are sent at a rate the receiver can process.

- Helps avoid data loss by managing how much data is sent at once.

Uses techniques like buffering to temporarily store data while the receiver processes it.

Example:

- Full buffer: Producer stops until consumer frees space.

- Vacancy: Consumer signals producer to send more data.

With a one-slot buffer, it's easier since the producer and consumer just check if the slot is full or empty.

Error Control at Data Link Layer:

Method 1:

  - If a frame is corrupted, it is discarded.

  - If a frame is good, it is delivered to the network layer.

Method 2:

  - If a frame is corrupted, it is discarded.

  - If a frame is good, an acknowledgment is sent to the sender.

- OR

Error Control:

- Corrupted Frame: Discarded.

- Good Frame: Delivered or Acknowledged.

Connectionless vs. Connection-Oriented:

- Connectionless: No relationship between frames. Each frame is independent.

- Connection-Oriented: Frames are numbered and sent in order, establishing a connection.

Data-Link Layer Protocols:

- Simple Protocol: Basic communication with no flow or error control.

- Stop-and-Wait Protocol: Sends one frame, waits for acknowledgment before sending the next.

- Go-Back-N Protocol: Sends multiple frames but resends all frames from an error point.

- Selective-Repeat Protocol: Only retransmits frames that were lost or corrupted.

The last two protocols are rarely used today.

A Finite State Machine (FSM) is a model that has a limited number of states. It behaves as follows:

- It stays in one state until an event occurs.

- When an event happens:

  - Actions are performed.

  - The machine moves to the next state based on the event.

FSM is used to model systems with clear, defined states and transitions.

OR

A Finite State Machine (FSM) is a system that:

- Has a limited number of states.

- Changes state based on events.

- Each event causes actions and moves to a new state.

The four traditional data-link layer protocols for managing flow and error control are:

- Simple Protocol: No flow or error control; frames sent continuously.

- Stop-and-Wait Protocol: Sender waits for acknowledgment after sending each frame.

- Go-Back-N Protocol: Allows multiple frames to be sent; retransmits all frames after an error.

- Selective-Repeat Protocol: Only retransmits specific erroneous frames, not all.

Data-Link Layer Protocols

- Simple Protocol – Sends data without waiting for a response (no flow/error control).

- Stop-and-Wait – Sends one frame, waits for acknowledgment (slow but reliable).

- Go-Back-N – Sends multiple frames, resends all from the error point (faster but wastes bandwidth).

- Selective-Repeat – Sends multiple frames, resends only the wrong one (efficient but complex).

These protocols control data flow and fix errors in communication.

Simple Protocol (Data-Link Layer)

- No flow control (no speed management).

- No error control (no retransmission if errors occur).

- Assumes receiver can handle all incoming frames without delay.

- Used in error-free and fast networks but not reliable for real-world communication.

FSM for Simple Protocol

For Sender:

- Idle → Waiting to send data.

- Send Frame → Sends a frame to the receiver.

- Repeat → Sends the next frame without waiting.

For Receiver:

- Waiting → Ready to receive a frame.

- Receive Frame → Gets and processes the frame.

- Repeat → Waits for the next frame (no acknowledgment needed).

Since there is no error checking or flow control, this works only in a perfect network with no data loss.

Example of Simple Protocol

Imagine a teacher giving a lecture in a classroom:

- The teacher (sender) keeps speaking without waiting to see if students (receiver) understand.

- The students (receiver) listen but do not give any feedback.

- The teacher does not stop or repeat anything, assuming students can follow everything.

In Simple Protocol, the sender keeps sending data without checking if the receiver is ready or if any data is lost. It works only if the receiver can handle everything perfectly.

Stop-and-Wait Protocol

- Uses flow & error control → Manages data speed and detects errors.

- Sends one frame at a time → Waits for acknowledgment before sending the next.

- Error detection → Uses CRC code to check for corrupted frames.

- If an error occurs → The sender resends the same frame.

This protocol is reliable but slow because it waits for acknowledgment after every frame.

Stop-and-Wait Protocol

- Flow and error control: The protocol controls how fast data is sent and ensures that errors are detected.

- One frame at a time: The sender sends one frame, waits for an acknowledgment from the receiver, and then sends the next frame.

- Error detection: Uses a CRC code to check if the frame is corrupted. If the frame is corrupted, it is resent.

Example of Stop-and-Wait Protocol

Imagine a postman (sender) delivering a letter to a person (receiver):

- The postman delivers one letter and waits until the person acknowledges they received it.

- If the letter is damaged or lost, the person will inform the postman.

- The postman will then resend the letter after receiving confirmation that the first one was either lost or corrupted.

In Stop-and-Wait Protocol, the sender waits for an acknowledgment before sending the next piece of data. This ensures reliable delivery but makes the process slow.

Piggybacking

- In both Simple and Stop-and-Wait protocols, data goes in one direction, and acknowledgments (ACKs) go in the other.

- Piggybacking means the receiver sends the ACK together with data when replying.

- This makes the system more efficient because it reduces the number of separate messages.

It helps to save time and resources by sending the ACK along with the data, rather than separately.

High-Level Data Link Control (HDLC)

- Bit-oriented protocol: HDLC works by sending data as a stream of bits rather than frames or bytes.

- Point-to-point and multipoint: Can be used for both direct connections (point-to-point) and network with multiple devices (multipoint).

- Implements Stop-and-Wait protocol: It sends one frame at a time and waits for acknowledgment before sending the next.

- Foundation for other protocols: Concepts in HDLC are used in other popular protocols like PPP, Ethernet, and wireless LANs.

HDLC is important because it provides a reliable and efficient way of communication that forms the basis for many modern networking protocols.

Configurations & Transfer Modes in HDLC

HDLC has two main transfer modes:

Normal Response Mode (NRM)

  - One device is the master, and the other is the slave.

  - The master controls the communication, and the slave only responds when asked.

Asynchronous Balanced Mode (ABM)

  - Both devices are equal and can send or receive data at any time.

  - There’s no master/slave; both can communicate freely.

These modes help decide how devices communicate in a network.

This makes the protocol reliable but can be slower because the sender has to wait after every frame.

Asynchronous Balanced Mode (ABM)

- Both devices are equal: Neither is in charge.

- Both can send and receive data at any time.

- No master/slave roles; both devices have the same power.

- Used in peer-to-peer communication, where devices need to talk freely.

ABM is for flexible and balanced communication between devices.

Framing in HDLC

HDLC has three types of frames:

Information Frames (I-frames)

  - Carry real data between devices.

  - Used to send actual information.

Supervisory Frames (S-frames)

  - Help with control and error fixing.

  - Used to manage the flow of data and ask for retransmission if needed.

Unnumbered Frames (U-frames)

  - Used for starting or ending the connection.

  - Handle tasks like link setup or shutdown.

Each frame type has a special purpose to manage data transfer, control, and connection.

HDLC Frames

HDLC frames organize data for communication. There are three types:

Information Frames (I-frames)

  - Carry actual data to be sent.

Supervisory Frames (S-frames)

  - Used to control data flow and fix errors (like asking for missing data).

Unnumbered Frames (U-frames)

  - Used for starting or ending the communication.

Each frame type helps manage how data is sent, controlled, and maintained.

Point-to-Point Protocol (PPP)

- Common for direct connections: PPP is used to connect devices directly, like computers to the Internet.

- Used by millions: Many people use PPP to connect their home computers to an Internet service provider (ISP).

- Manages data transfer: PPP makes sure data is sent correctly, controls the flow, and checks for errors.

- Important for reliable connections: It helps keep internet connections secure and working smoothly.

PPP is a simple and reliable way to connect to the internet and manage communication between devices.

Services Provided by PPP

PPP offers several useful services for direct connections:

- Error Detection: It checks for mistakes in data and asks to resend if something is wrong.

- Flow Control: It manages the speed of data to avoid overloading the receiver.

- Authentication: PPP can verify that devices are who they say they are using passwords.

- Encapsulation: It can send different types of data like IP and IPX.

- Link Quality Monitoring: It checks if the connection is good and can disconnect if it's poor.

PPP keeps things simple but still provides the necessary features for reliable communication.

Point-to-Point Protocol (PPP)

- Most common for direct connections: PPP is the main protocol used to connect devices, like a home computer to the internet.

- Used by millions: Many people use PPP to connect to their internet service provider (ISP).

- Controls data transfer: PPP makes sure data is sent correctly by checking for errors and controlling the flow of information.

PPP helps connect devices and ensures smooth and reliable communication.

PPP Frame Format

PPP sends data in bytes and each frame has these parts:

- Flag: Marks the start and end of the frame.

- Address: Tells the frame's destination.

- Control: Helps manage the frame.

- Protocol: Shows what type of data is inside.

- Data: The actual information being sent.

- FCS: Used to check for errors in the data.

This structure makes it easy to send and check data properly.

Multiplexing in PPP

PPP uses other protocols to set up the connection, verify devices, and send data across different networks. These include:

Link Control Protocol (LCP)

  - Helps create and set up the link between devices.

Authentication Protocols (APs)

  - PAP and CHAP are used to verify the identity of devices before they can communicate.

Network Control Protocols (NCPs)

  - These help manage and configure network protocols like IP over the PPP connection.

These protocols make PPP flexible, allowing it to work with different networks securely.

Multiplexing in PPP

In PPP, multiplexing means sending different types of data over the same connection. It uses these protocols:

- Link Control Protocol (LCP): Sets up and manages the connection.

- Authentication Protocols (APs): PAP and CHAP make sure the devices are trusted before they connect.

- Network Control Protocols (NCPs): Help manage and set up network data like IP.

This way, PPP can carry different data types securely over one connection.

LCP Packet Encapsulated in a Frame

In PPP, the Link Control Protocol (LCP) packet is encapsulated within a PPP frame. This is how it works:

- LCP Packet: Contains information used to configure, test, and maintain the connection between two devices.

- PPP Frame: This frame wraps the LCP packet along with other data like address, control, and FCS to be sent across the network.

The LCP packet is included in the data section of the PPP frame, allowing devices to negotiate settings and manage the connection while data flows.

In simple terms, the LCP packet is a message inside the PPP frame that helps set up and manage the connection.\

LCP Packets

LCP (Link Control Protocol) packets are used in PPP to set up, manage, and maintain the connection between devices. There are different types of LCP packets:

- Configure Request (ConfReq): One device asks the other to agree on connection settings.

- Configure Acknowledge (ConfAck): The other device agrees to the settings.

- Configure Nak (ConfNak): The device suggests changes to the settings.

- Configure Reject (ConfReject): The device completely rejects the settings.

- Terminate Request (TermReq): A request to end the connection.

- Terminate Acknowledge (TermAck): The device confirms that the connection is ended.

- Code Reject (CodeReject): Indicates an error in the LCP packet received.

These LCP packets help devices negotiate, manage, and close the connection properly.

Multiplexing in PPP

PPP uses extra protocols to set up the connection, verify devices, and carry network data. These include:

Link Control Protocol (LCP):

  - Helps create and manage the connection between devices.

Authentication Protocols (APs):

  - PAP and CHAP make sure the devices are trusted before they communicate.

Network Control Protocols (NCPs):

  - Help manage network data like IP over the PPP connection.

These protocols let PPP handle different types of data and ensure security over the connection.

Authentication Protocols in PPP

PPP uses Authentication Protocols to make sure only trusted devices can connect to the network. There are two main methods:

PAP (Password Authentication Protocol):

  - A simple method where the username and password are sent as plain text. This method is not very secure because the information is not encrypted.

CHAP (Challenge Handshake Authentication Protocol):

  - A more secure method. The server sends a random challenge, and the device responds with an encrypted value of the password and challenge. This way, passwords are never sent openly.

These protocols help ensure that only authorized devices can access the network.

Authentication Protocols in PPP

PPP uses Authentication Protocols to make sure that only trusted devices can connect. The two main methods are:

PAP (Password Authentication Protocol):

  - A simple method where the username and password are sent as plain text. It’s less secure because the information is not hidden or encrypted.

CHAP (Challenge Handshake Authentication Protocol):

  - A more secure method where the device is asked to respond to a random challenge with an encrypted value. This keeps the password safe because it’s never sent openly.

These protocols help make sure only authorized devices can access the network.

CHAP Packets in a PPP Frame

When using CHAP (Challenge Handshake Authentication Protocol) in PPP, the CHAP packet is sent inside a PPP frame to the server for authentication. Here's how it works:

CHAP Packet:

  - The server sends a random number (challenge) to the client.

  - The client replies with an encrypted response (hash) using the challenge and the password.

  - The server checks the response to see if it matches its own calculation. If it does, the client is authenticated.

PPP Frame:

  - The CHAP packet is placed inside the data part of the PPP frame, which also contains other information like address and error-checking.

In short, the CHAP packet inside a PPP frame ensures the client’s identity is checked securely without sending passwords openly.

IP Datagram in a PPP Frame

In PPP (Point-to-Point Protocol), an IP datagram (a packet of data) is placed inside a PPP frame to send over the network. Here's how it works:

- The IP datagram contains the actual data (like web pages or files).

- The PPP frame wraps the IP datagram with extra information (like headers and error checks) to help it reach the right destination.

In simple words, the IP datagram is the data you want to send, and the PPP frame is the package that carries it safely through the network.

When nodes (devices) are connected through a multipoint or broadcast link, meaning they all share the same connection, they need a multiple-access protocol to manage how they take turns using the link.

These protocols help decide who can send data at any given time, preventing conflicts like two devices trying to send data at the same time. Many different protocols have been developed to handle this access control.

All these protocols are part of the Media Access Control (MAC) sub-layer, which is a part of the Data Link Layer. The MAC sub-layer ensures that the devices can send and receive data smoothly on shared links.

Types of Multiple-Access Protocols (Very Simple)

When many devices share a network, multiple-access protocols help them send data without interfering with each other.

Channel Partitioning Protocols – Each device gets its own turn.

  - TDMA: Devices send data at different times.

  - FDMA: Devices use different frequencies.

  - CDMA: Devices send data at the same time but with unique codes.

Random Access Protocols – Devices send whenever they want but must avoid collisions.

  - Aloha: Send data and wait for a reply.

  - Slotted Aloha: Send only at fixed time slots.

  - CSMA: Listen first before sending.

    - CSMA/CD: Stop if a collision happens.

    - CSMA/CA: Wait before sending to avoid collisions.

Controlled Access Protocols – A system decides who can send data.

  - Polling: A central device asks each device if it has data to send.

  - Token Passing: Only the device with the "token" can send data.

These methods help devices take turns and avoid sending at the same time.