Sample Paper

FINALTERM EXAMINATION

Fall 2022

CS601 - Data Communication

Time: 90 min

Marks: 60

Question No: 1      (Marks: 01) - Please choose the correct option

Bluetooth technology is the implementation of a protocol defined by the _________ standard.

- IEEE 802.16

- IEEE 802.18

- IEEE 802.17

- IEEE 802.15

Question No: 2      (Marks: 01) - Please choose the correct option

IEEE Project 802 is a way of specifying functions of the physical layer and the data-link layer of major _______________.

- LAN protocols

- WAN modules

- MAN protocols

- Parts of OSI model

Question No: 3      (Marks: 01) - Please choose the correct option

At the CRC checker, a non-zero reminder means that the ______ is damaged.

- Data word

- Keyword

- Parity bit

- Output

Question No: 4      (Marks: 01) - Please choose the correct option

The data link layer takes the packets from _________ and encapsulates them into frames for transmission.

- Network layer

- Physical layer

- Transport layer

- Application layer

Question No: 5      (Marks: 01) - Please choose the correct option

Framing and error correction is a function performed on _____________ layer of OSI model.

- Network layer

- Physical layer

- Datalink layer

- Presentation layer

Question No: 6      (Marks: 01) - Please choose the correct option

In context of IEEE 802.11, BSS stands for ____________.

- Basic Service Set

- Basic Service Solution

- Basic Solution Setting

- Basic Solution Sorting

Question No: 7      (Marks: 01) - Please choose the correct option

The ________LAN was developed in the 1970s.

- Internet

- Ethernet

- Intranet

- Protocol

Question No: 8      (Marks: 01) - Please choose the correct option

In order to detect five errors, the Hamming distance between each pair of code words should be _________.

- 4

- 5

- 6

- 7

Question No: 9      (Marks: 01) - Please choose the correct option

Which sub layer of the data link layer performs data link functions that depend upon the type of medium?

- Logical link control sub layer

- Media access control sub layer

- Network interface control sub layer

- Error control sub layer

Question No: 10      (Marks: 01) - Please choose the correct option

In point-to-point link, ______________ sub-layer is not present in data link layer.

- Data Link control

- Media Authentication Control

- Data Division Control

- Media Access Control

Question No: 11      (Marks: 01) - Please choose the correct option

Redundant switches create _________ in the system.

- Loops

- Reset points

- Bugs

- Flow

Question No: 12      (Marks: 01) - Please choose the correct option

Ethernet is ___________like IP and UDP

- Unreliable

- Reliable

- Flexible

- Fast

Question No: 13      (Marks: 01) - Please choose the correct option

Flags are used as text characters in _______.

- Bit Oriented Framing

- Byte Oriented Framing

- Multiplexing

- Interleaving

Question No: 14      (Marks: 01) - Please choose the correct option

Automatic repeat request error management mechanism is provided by ________.

- Logical link control sub layer

- Media access control sub layer

- Network interface control sub layer

- Error control sub layer

Question No: 15      (Marks: 01) - Please choose the correct option

Data-Link layer needs to pack _________ into _____________.

- Bits, frames

- Frames, bits

- Bytes, frames

- Frames, bytes

Question No: 16      (Marks: 01) - Please choose the correct option

A ____________ has a physical and logical (IP) address for each of its interfaces

- Switch

- Hub

- Router

- Bridge

Question No: 17      (Marks: 01) - Please choose the correct option

Services offered by Telephone companies’ ___________LATAs are called Inter-LATA services.

- Inside

- Outside

- Between

- Through

Question No: 18      (Marks: 01) - Please choose the correct option

Data link control (DLC) layer provides services between two adjacent nodes either the link is _______.

- Dedicated

- Broadcast

- Both Dedicated & Broadcast

- None of the given

Question No: 19      (Marks: 01) - Please choose the correct option

Header of a frame generally contains ______________

- Synchronization bytes

- Addresses

- Frame identifiers

- All of the mentioned

Question No: 20      (Marks: 01) - Please choose the correct option

____________ device operates on physical layer.

- Switch

- Hub

- Router

- Bridge

Question No: 21      (Marks: 01) - Please choose the correct option

In SONET, frames of lower rate can be synchronously time-division __________into a higher-rate frame

- Multiplexed

- Duplex

- Multiplied

- Merged

Question No: 22      (Marks: 01) - Please choose the correct option

CRC stands for __________.

- cyclic redundancy check

- code repeat check

- code redundancy check

- cyclic repeat check

Question No: 23      (Marks: 01) - Please choose the correct option

Framing separates a message by ________ a sender address and a destination address.

- Multiplying

- Adding

- Dividing

- Subtracting

Question No: 24      (Marks: 01) - Please choose the correct option

In data link flow control, if frames are produced faster than they are consumed at the receiving data link layer, the frames will be ____________.

- Added

- Discarded

- Stored

- Forwarded

Question No: 25      (Marks: 01) - Please choose the correct option

At Data link Layer, if frame is corrupted, it is silently discarded and if it is good, an acknowledgement is sent to sender. It is known as ____________ technique.

- Address Configuration

- Error Control

- Link Establishment

- Flow Control

Question No: 26      (Marks: 01) - Please choose the correct option

The term FTTH stands for __________.

- Fiber technology to home

- Fiber to the home

- Free transmission to home

- Fiber transmission to home

Question No: 27      (Marks: 01) - Please choose the correct option

Which of the following is the multiple access protocol for channel access control?

- CSMA/CD

- CSMA/CA

- Both CSMA/CD & CSMA/CA

- HDLC

Question No: 28      (Marks: 01) - Please choose the correct option

ALOHA, the earliest random-access method, was developed in early __________.

- 1950’s

- 1970’s

- 1980’s

- 1990’s

Question No: 29      (Marks: 01) - Please choose the correct option

In fiber to the curb communication, Tap or coax drop is used for _______transmission to home subscribers.

- Analog TV

- Voice over IP

- Data transmission

- Fiber optic signals

Question No: 30      (Marks: 01) - Please choose the correct option

In medium access control, to minimize the chance of data collision and, therefore, increase the performance, ________ was developed.

- CDMA

- TDMA

- CSMA

- FDMA

Question No: 31      (Marks: 01) - Please choose the correct option

In ________layer, Bluetooth uses FHSS method in order to avoid interference from other devices.

- Physical

- Datalink

- Network

- Application

Question No: 32      (Marks: 01) - Please choose the correct option

_______ is the temporary storage of data frames at sending-end receiving stations.

- Buffers

- Physical Media

- Routers

- Data Link Layer

Question No: 33      (Marks: 01) - Please choose the correct option

How error detection and correction is done?

- By passing it through equalizer

- By passing it through filter

- By amplifying it

- By adding redundancy bits

Question No: 34      (Marks: 01) - Please choose the correct option

Which of the following is a type of Random-Access Protocol?

- FDMA

- CSMA/CA

- CDMA

- TDMA

Question No: 35      (Marks: 01) - Please choose the correct option

RTS value of subfield in ______ frame is 1100.

- Control

- Management

- Data

- Payload

Question No: 36      (Marks: 01) - Please choose the correct option

Which of the following error detection technique is more efficient?

- Parity check

- Cyclic redundancy check

- Parity & Cyclic redundancy check

- Checksum

Question No: 37      (Marks: 01) - Please choose the correct option

The Internet has been using a checksum of ___________.

- 2 bits

- 4 bits

- 8 bits

- 16 bits

Question No: 38      (Marks: 01) - Please choose the correct option

While using control access protocol, in the ____________ method, the stations in a network are organized in a logical ring.

- Token-passing

- Polling

- Reservation

- Aloha

Question No: 39      (Marks: 01) - Please choose the correct option

______ eliminates collision and connects heterogeneous devices.

- Switch

- Hub

- Router

- Repeater

Question No: 40      (Marks: 01) - Please choose the correct option

In Address Resolution Protocol (ARP), a packet is encapsulated directly into a ________.

- Data link integer

- Network frame

- Network station

- Data link frame

Question No: 41      (Marks: 03)

Wireless LAN works on two layers of TCP/IP model. You are required to write down those layers name.

Question No: 42      (Marks: 03)

Data link layer achieved its functionality by further dividing into sublayers. You are required to mention the names of these sublayers.

Question No: 43      (Marks: 03)

Which of the following are not the services provided by Point-to-Point Protocol:

Framing, Data Exchange, Flow Control, Authentication, Error Detection, Error Correction, Multi link Addressing

Question No: 44      (Marks: 03)

There are three types of Frames defined by IEEE 802.11 specification for Wireless LANs. Which frame is responsible for initial communication between access points?

Question No: 45      (Marks: 03)

Chunk Interleaving falls in what category: Error correction or Error Detection?  Also mention what type of errors the Chunk Interleaving works on?

Question No: 46      (Marks: 03)

Write names of three channelized protocols exists under the Multiple-Access Protocol.

Question No: 47      (Marks: 05)

Write the number of components of the telephone network? Enlist the major components of Telephone network also.

Question No: 48      (Marks: 05)

Internet Protocol Control Protocol (IPCP) is a common protocol of the Network Control Protocol (NCP) that possess code values that actually goes in the code field of the IPCP packet. Considering this, given below is a table of IPCP packet, you are required to mention the IPCP codes of the respective packets.

| IPCP Packet | IPCP Codes |
| --- | --- |
| Configure-Ack/ Acknowledgement |  |
| Code-Reject |  |
| Configure-Request |  |
| Configure-nak |  |
| Terminate-ack |  |

Question No: 49      (Marks: 05)

During the evolution process, Ethernet has gone through four generations. Enlist all these generations.

Question No: 50      (Marks: 05)

Hamming distance is used for error detection. Considering this you are required to answer the following cases:

i) Write down the formula to find minimum hamming distance.

ii) Number of bits required to correct 10 bits using hamming distance formula?